Mini-Control (IR/R)

for

Mosconi DSPs

(not GladenOne120.4 DSP)

Users Manual

1. Connection:

Connect the Mini-Control to the Mosconi DSP. Only the provided RJ11 lead or an original Mosconi RCD/RTC lead is supported.

2. First start-up:

Turn on the DSP. Three seconds after switching on the Mini-Control synchronizes with the DSP, i.e. the DSP inherit the settings for volume, subwoofer level and preset from the the Mini-Control according to the potentiometer’s positions. As from now on the DSP boots with the last settings loaded.

3. Operation

Operate the DSP via the Mini-Control. While switching the presets the active one is displayed by flashing the preset LED.

4. Disconnect the Mini-Control

In case the DSP shall be used without the Mini-Control you can reset the settings within Mosconi software using the button „Load factory settings“.

Operate using an IR-remote control

The Mini-Control can interpret the codes of almost any standard off-the-shelf remote control. An IR remote control (wireless) or an OEM steeringwheel remote control adapter (wired, 3,5mm phone jack) can be used. As default the codes of the Alpine RUE-4202 are setup.

While remote control operation the position of the potentiometer sets the start-up volume, respectively the start-up subwoofer level. Using the remote control the start-up settings can be modified step-by-step. Button 6 (shift mode) switches the function level. As long as the function level 2 is active the preset LED glows continuously and the functions “Balance”, “Fader” and “All Reset” can be used. The Mini-Control switches back to function level 1 after a time out of three seconds after the last action. Function “Reset” activates the start-up settings. Function “All Reset” resets “Balance” and “Fader” to the center position additionally.

Reset settings

Trained IR codes as well as all start-up settings can be reset to factory defaults. For this purpose press and hold the potentiometer for three seconds while turning on the DSP. The Preset LED flashes rapidly to confirm the reset of all settings.

Training of IR codes (Version IR)

The IR codes can be trained if you plan to use a remote control with codes differing from the default setup. To start the training process press and hold the potentiometer for three seconds while the DSP is running until the Preset LED glows. Note: The current volume at the beginning of the training is saved as mute volume! After the Preset LED went out press the button for function 1 (volume +). The Preset LED flashes once for confirmation. Repeat for all eleven functions (see table below). If a function is not to be used push the potentiometer to skip a single function. After training all eleven functions the Preset LED flashes three times and the training process is finished.

| 1 | Volume + |
| --- | --- |
| 2 | Volume - |
| 3 | Subwoofer Level + / Fader front |
| 4 | Subwoofer Level - / Fader Rear |
| 5 | Preset |
| 6 | Shift mode |
| 7 | Mute |
| 8 | Reset / All Reset |
| 9 | Balance right |
| 10 | Balance left |
| 11 | Reserve / no function |

Training of voltage values (Version R)

The Mini-Control can interpret the voltage values of a remote control adapter with resistance coding. The resistance coding is often used at remote control adapters for Car-PCs. By default the voltage values of the adapters Audiotechnik Dietz (65060) and CarPC Joycon Exr are set up. New voltage values can be trained if an adapter with different values is to be used. To start the training process press and hold the potentiometer for three seconds while the DSP is running until the Preset LED glows. Note: The current volume at the beginning of the training is saved as mute volume! After the Preset LED went out press the button for function 1 (volume +). The Preset LED flashes once for confirmation. Repeat for all five functions (see table below). If a function is not to be used push the potentiometer to skip a single function. After training all five functions the Preset LED flashes three times and the training process is finished.

| 1 | Volume + |
| --- | --- |
| 2 | Volume - |
| 3 | Sublevel + / Fader Front |
| 4 | Subwoofer Level - / Fader Rear |
| 5 | Preset |

Adaption using resistance values – basic circuit